### Hydrostatics - ContainerGroup6

Stability 24.00.04.133, build: 133

Model file: C:\Users\User\Downloads\ContainerGroup6 (4) (Highest precision, 34 sections, Trimming off, Skin thickness not applied). Long. datum: AP; Vert. datum: Baseline. Analysis tolerance - ideal(worst case): Disp.%: 0.01000(0.100); Trim%(LCG-TCG): 0.01000(0.100); Heel%(LCG-TCG): 0.01000(0.100)

Damage Case - Intact

Fixed Trim = 0 m (+ve by stern)

Specific gravity = 1.025; (Density = 1.025 tonne/m^3)

| Draft Amidships m | 4.000 | 5.063 | 6.103 | 7.128 | 8.141 | 9.143 | 10.134 | 11.115 | 12.086 | 13.048 | 14.000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Displacement t | 28569 | 37265 | 45962 | 54658 | 63355 | 72051 | 80747 | 89444 | 98140 | 106837 | 115533 |
| Heel deg | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Draft at FP m | 4.000 | 5.063 | 6.103 | 7.128 | 8.141 | 9.143 | 10.134 | 11.115 | 12.086 | 13.048 | 14.000 |
| Draft at AP m | 4.000 | 5.063 | 6.103 | 7.128 | 8.141 | 9.143 | 10.134 | 11.115 | 12.086 | 13.048 | 14.000 |
| Draft at LCF m | 4.000 | 5.063 | 6.103 | 7.128 | 8.141 | 9.143 | 10.134 | 11.115 | 12.086 | 13.048 | 14.000 |
| Trim (+ve by stern) m | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| WL Length m | 307.358 | 306.590 | 305.572 | 304.519 | 304.258 | 305.936 | 306.242 | 306.834 | 307.475 | 308.094 | 308.684 |
| Beam max extents on WL m | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.552 |
| Wetted Area m^2 | 9125.259 | 9829.736 | 10502.412 | 11148.220 | 11794.567 | 12465.653 | 13132.409 | 13766.724 | 14387.673 | 15001.408 | 15610.633 |
| Waterpl. Area m^2 | 7879.895 | 8077.460 | 8219.180 | 8327.579 | 8417.386 | 8512.788 | 8606.456 | 8691.570 | 8776.372 | 8863.429 | 8953.369 |
| Prismatic coeff. (Cp) | 0.662 | 0.683 | 0.700 | 0.714 | 0.725 | 0.730 | 0.737 | 0.743 | 0.748 | 0.752 | 0.757 |
| Block coeff. (Cb) | 0.656 | 0.678 | 0.696 | 0.711 | 0.722 | 0.727 | 0.735 | 0.741 | 0.746 | 0.750 | 0.755 |
| Max Sect. area coeff. (Cm) | 0.991 | 0.993 | 0.994 | 0.995 | 0.996 | 0.996 | 0.997 | 0.997 | 0.997 | 0.997 | 0.998 |
| Waterpl. area coeff. (Cwp) | 0.742 | 0.763 | 0.779 | 0.792 | 0.801 | 0.805 | 0.813 | 0.820 | 0.826 | 0.833 | 0.839 |
| LCB from zero pt. (+ve fwd) m | 145.020 | 145.858 | 146.462 | 146.891 | 147.146 | 147.195 | 147.041 | 146.791 | 146.516 | 146.240 | 145.973 |
| LCF from zero pt. (+ve fwd) m | 148.307 | 148.866 | 149.163 | 149.049 | 148.291 | 146.657 | 144.991 | 144.015 | 143.378 | 142.924 | 142.584 |
| KB m | 2.127 | 2.689 | 3.238 | 3.776 | 4.306 | 4.831 | 5.349 | 5.863 | 6.372 | 6.876 | 7.377 |
| KG m | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 | 13.000 |
| BMt m | 21.264 | 16.952 | 14.149 | 12.177 | 10.717 | 9.600 | 8.729 | 8.024 | 7.439 | 6.947 | 6.530 |
| BML m | 1503.078 | 1219.747 | 1028.134 | 889.332 | 784.468 | 707.433 | 647.184 | 597.552 | 557.366 | 524.644 | 497.733 |
| GMt m | 10.391 | 6.641 | 4.387 | 2.953 | 2.023 | 1.431 | 1.078 | 0.886 | 0.810 | 0.824 | 0.907 |
| GML m | 1492.205 | 1209.436 | 1018.372 | 880.107 | 775.774 | 699.264 | 639.534 | 590.415 | 550.737 | 518.520 | 492.110 |
| KMt m | 23.391 | 19.641 | 17.387 | 15.953 | 15.023 | 14.431 | 14.078 | 13.886 | 13.810 | 13.824 | 13.907 |
| KML m | 1505.205 | 1222.436 | 1031.372 | 893.107 | 788.774 | 712.264 | 652.534 | 603.415 | 563.737 | 531.520 | 505.110 |
| Immersion (TPc) tonne/cm | 80.769 | 82.794 | 84.247 | 85.358 | 86.278 | 87.256 | 88.216 | 89.089 | 89.958 | 90.850 | 91.772 |
| MTc tonne.m | 1406.963 | 1487.459 | 1544.762 | 1587.626 | 1622.075 | 1662.791 | 1704.311 | 1742.872 | 1783.810 | 1828.281 | 1876.401 |
| RM at 1deg = GMt.Disp.sin(1) tonne.m | 5180.861 | 4319.188 | 3518.947 | 2816.789 | 2236.661 | 1799.313 | 1519.657 | 1383.737 | 1387.904 | 1535.500 | 1829.124 |
| Max deck inclination deg | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Trim angle (+ve by stern) deg | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |