### Hydrostatics - Group6

Stability 24.00.04.133, build: 133

Model file: C:\Users\sanja\OneDrive\IIT_KGP\3RD yr\Laboratory\Cad-cam\Cad-cam\Group6 (Highest precision, 506 sections, Trimming off, Skin thickness not applied). Long. datum: AP; Vert. datum: Baseline. Analysis tolerance - ideal(worst case): Disp.%: 0.01000(0.100); Trim%(LCG-TCG): 0.01000(0.100); Heel%(LCG-TCG): 0.01000(0.100)

Intact

Fixed Trim = 0 m (+ve by stern)

Specific gravity = 1.025; (Density = 1.025 tonne/m^3)

| Draft Amidships m | 14.500 | 13.500 | 12.500 | 11.500 | 10.500 | 9.500 | 8.500 | 7.500 | 6.500 | 5.500 | 4.500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Displacement t | 120372 | 111097 | 101947 | 92923 | 84029 | 75243 | 66538 | 57906 | 49357 | 40917 | 32627 |
| Heel deg | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Draft at FP m | 14.500 | 13.500 | 12.500 | 11.500 | 10.500 | 9.500 | 8.500 | 7.500 | 6.500 | 5.500 | 4.500 |
| Draft at AP m | 14.500 | 13.500 | 12.500 | 11.500 | 10.500 | 9.500 | 8.500 | 7.500 | 6.500 | 5.500 | 4.500 |
| Draft at LCF m | 14.500 | 13.500 | 12.500 | 11.500 | 10.500 | 9.500 | 8.500 | 7.500 | 6.500 | 5.500 | 4.500 |
| Trim (+ve by stern) m | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| WL Length m | 308.592 | 307.307 | 305.893 | 304.398 | 303.490 | 304.175 | 305.899 | 307.253 | 308.397 | 309.258 | 308.399 |
| Beam max extents on WL m | 34.554 | 34.552 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 | 34.550 |
| Wetted Area m^2 | 15971.999 | 15318.542 | 14655.203 | 13987.476 | 13311.159 | 12654.947 | 12017.109 | 11386.983 | 10755.707 | 10115.272 | 9457.473 |
| Waterpl. Area m^2 | 9108.460 | 8989.699 | 8863.360 | 8740.127 | 8619.090 | 8529.052 | 8457.564 | 8384.176 | 8292.577 | 8168.960 | 7998.434 |
| Prismatic coeff. (Cp) | 0.761 | 0.758 | 0.755 | 0.752 | 0.747 | 0.738 | 0.725 | 0.713 | 0.699 | 0.683 | 0.669 |
| Block coeff. (Cb) | 0.759 | 0.756 | 0.752 | 0.749 | 0.744 | 0.735 | 0.722 | 0.709 | 0.694 | 0.678 | 0.663 |
| Max Sect. area coeff. (Cm) | 0.998 | 0.997 | 0.997 | 0.997 | 0.997 | 0.996 | 0.996 | 0.995 | 0.995 | 0.994 | 0.992 |
| Waterpl. area coeff. (Cwp) | 0.854 | 0.847 | 0.839 | 0.831 | 0.822 | 0.812 | 0.800 | 0.790 | 0.778 | 0.765 | 0.751 |
| LCB from zero pt. (+ve fwd) m | 146.209 | 146.481 | 146.753 | 146.989 | 147.130 | 147.140 | 147.017 | 146.759 | 146.366 | 145.832 | 145.128 |
| LCF from zero pt. (+ve fwd) m | 142.834 | 143.111 | 143.845 | 144.934 | 146.385 | 147.624 | 148.480 | 148.947 | 149.044 | 148.819 | 148.349 |
| KB m | 7.645 | 7.114 | 6.586 | 6.060 | 5.537 | 5.016 | 4.494 | 3.972 | 3.447 | 2.920 | 2.391 |
| KG m | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 | 13.400 |
| BMt m | 6.370 | 6.765 | 7.221 | 7.762 | 8.413 | 9.230 | 10.262 | 11.584 | 13.315 | 15.656 | 18.986 |
| BML m | 502.394 | 525.890 | 551.885 | 583.578 | 622.367 | 678.469 | 754.590 | 852.609 | 977.569 | 1140.427 | 1362.216 |
| GMt m | 0.615 | 0.479 | 0.407 | 0.422 | 0.550 | 0.845 | 1.356 | 2.156 | 3.362 | 5.176 | 7.977 |
| GML m | 496.639 | 519.604 | 545.071 | 576.238 | 614.504 | 670.084 | 745.685 | 843.181 | 967.617 | 1129.947 | 1351.207 |
| KMt m | 14.015 | 13.879 | 13.807 | 13.822 | 13.950 | 14.245 | 14.756 | 15.556 | 16.762 | 18.576 | 21.377 |
| KML m | 510.039 | 533.004 | 558.471 | 589.638 | 627.904 | 683.484 | 759.085 | 856.581 | 981.017 | 1143.347 | 1364.607 |
| Immersion (TPc) tonne/cm | 93.362 | 92.144 | 90.849 | 89.586 | 88.346 | 87.423 | 86.690 | 85.938 | 84.999 | 83.732 | 81.984 |
| MTc tonne.m | 1972.976 | 1905.159 | 1833.937 | 1767.190 | 1704.159 | 1664.007 | 1637.512 | 1611.394 | 1576.192 | 1525.878 | 1454.956 |
| RM at 1deg = GMt.Disp.sin(1) tonne.m | 1291.506 | 928.313 | 723.434 | 684.954 | 806.507 | 1110.010 | 1575.178 | 2178.353 | 2895.783 | 3696.224 | 4542.471 |
| Max deck inclination deg | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Trim angle (+ve by stern) deg | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |